Project 6 Elevator Setup Scripts (RPi)

Introduction:

The Elevator Project uses a Raspberry Pi (RPi) computer as a supervisory controller, and it must perform a variety of different tasks to complete its duties within the elevator system. The RPi must communicate with the car, floor, and elevator controllers over a shared CAN buss using the PEAK CAN driver and adapter; process requests, log system information and request records in a MySQL database; host the database and a website on the local network to enable remote control of the system using a web-based GUI. All of these functions require supporting architecture and setup in order to function optimally, and the setup process is fairly involved. Because of the length of the setup process, it is possible that some required settings are missed and go unconfigured upon system setup. Since we are sharing the physical hardware platform with another group, we should try to work within a different environment on the RPi as much as we can, and we cannot be sure that all of the settings remain in the required state. For these reasons, we decided to script the setup process for each of the foundational steps (the RPi setup, server setup, and database setup) using Bash to ensure that our working environment remains up to date, properly configured, and that it is easy to refresh if something goes wrong.

Equipment Description:

The RPi that we will be using is an RPi 3B, which will be connected to the provided wireless elevator network, and by CAN to the floor, car, and elevator controllers. Most of the setup steps that are performed in these setup scripts install dependency packages, establish filesystems, create or modify configuration files, or prompts the user to make adjustments where it is easier than automation. The scripts will set up the RPi environment, set up MySQL database, and configure the apache2 server for hosting the website and database.

Setup Scripts:

The supervisory controller is the main computational element in the elevator system; it manages the system’s overall function. The supervisory controller:

- Implements a finite state machine to direct the elevator car to a desired floor based on the system’s current state and a user request.

- Receives input from the car controller and floor controllers when a hardware request is given, and instructs the elevator controller how to articulate the elevator winch.

- Receives input from the elevator controller reporting the car’s current position in the system.

- Dispatch the elevator car via the elevator controller, informed by local and remote requests.

- Communicate with the remote control system (GUI) using a locally-hosted MySQL database, where it will record information about hardware request, current system state, and request service completion.

Elevator controller:

The elevator controller will:

- Receive instructions from the supervisory controller regarding floor requests originating from the floor controllers, car controller, and the GUI.

- Broadcast the car’s current position over the CAN network to all available nodes.

The elevator controller will not:

- Communicate directly with the GUI.

Car Controller:

The car controller will:

- Designate a floor that the elevator car should be sent to by interpreting controller-local button presses.

- Display the selected floor using an LED indicator.

- Send requests to the supervisory controller.

Floor Controllers:

The floor controllers will:

- Send requests to the supervisory controller to ask that the elevator car be brought to its floor.

- Display that a request has been submitted and is being served using an LED.

Graphical User Interface (GUI):

The graphical user interface is an alternate, remote control method for the elevator system which functions in parallel with the local request system (the combination of the floor controllers and the car controller). The supervisory controller manages competing requests between the local and remote request systems, prioritizing local requests. The GUI will:

- communicate with the supervisory controller through the MySQL database.

- Both send inputs and receive feedback from the database.

- Display car movement and service status using on-screen indicators.

GUI Design Decisions:

Since the GUI is a remote application, hosted on the project website, it is assumed that the user will not have access to the local request system or indicators, so the GUI should reproduce the function of the local request system. This means that the GUI should offer both car controller input options and indicators, and floor controller input options and indicators. The GUI will allow a user to select a chosen floor request, having the car sent to that floor as if the indicated floor’s hardware call button had been pressed. It should display an indication that the request was received, by way of a visual change to the GUI, lighting the selected floor’s button in the time between the request submission and the service receipt through the database. The user will subsequently be able to request another floor to where the car should be sent. The GUI will also indicate the car’s direction of travel, and allow users to select an adjacent floor using the direction buttons. The GUI will monitor the physical state of the elevator system through the database, which is enabled by the supervisory controller’s data-logging.